HD63B09EP Technical Reference Guide

By Chet Simpson

Modifications and Corrections by Alan DeKok

Additional Corrections by Roger Taylor

Copyright (C) 1994 Chet Simpson and Alan DeKok

All Rights Reserved.

License: This document may be freely distributed in electronic

form as long as it is unchanged, and the copyright notice

is intact.  Permission is NOT given to reproduce it in

any other form without the written consent of the authors.

INDEX

Introduction.................................................1

Summary of Features..........................................1

Description of Additional Registers..........................2

Modes of Operation...........................................3

Native Mode and Timing Loops.................................3

Modes of the Fast Interrupt Request (FIRQ)...................4

Inter-Register Instructions..................................4

Bit Manipulation of Memory Locations.........................4

Bit Transfers Between Memory Locations and Registers.........5

Block Transfers..............................................6

New math instructions (MULD, DIVD, DIVQ).....................7

Error Trapping...............................................7

Additional instructions......................................7

OP-Code Table...............................................10

Mnemonic Table..............................................19

Branch Instructions........................................24

Bit Manipulation and Transfers.............................24

Logical Memory Instructions................................25

Inter-Register Instructions................................25

Index Adressing Modes and Post- Byte Information...........26

Register Description.......................................27

Push/Pull Order............................................27

Push/Pull Post-Byte........................................27

Condition Code Register....................................27

HD63B09EP Technical Reference Guide                                  Page  1

Introduction

The  HD63B09EP  microprocessor  by  Hitachi, is a MC68B09E compatible

chip containing additional registers and an  additional  instruction  set.

The  6309  was  thought  to  be  a  flakey  chip  though, because it would

sometimes crash or change the values of registers when it  encountered  an

addressing  mode or opcode invalid to the 6809. This was later found to be

an extended instruction set and a feature that would trap some programming

errors and jump to a specified location in memory.

Hitachi  licensed  the  rights  of  the  6809  instruction  set  from

Motorola to make a 6809 compatible chip. When they  finished  the  design,

they  found there was a lot of unused space in the chip. With this in mind

they added extra registers and expanded on the instruction set, but due to

the  licensing  agreement  with  Motorola, they were unable to release the

information about the extra features.

Not only does the chip have an expanded instruction set, but  it also

has  a  native mode that will run many of the instructions in  fewer clock

cycles and a mode select for the FIRQ  (Fast Interrupt ReQuest) that  will

enable it to opperate the same as the IRQ.

In fact, all new  instructions will execute  in emulation mode, which

was originally seen  when 'illegal' 6809 instructions produced odd effects

when run on a computer with a 6309 installed.

The  additional  instruction set was first written about in the April

1988 issue of "Oh!FM", a Japanese magazine, and was  later  translated  by

Hirotsugu  Kakagawa.  This  opened a whole new door to those who wished to

use the 6309 in place of the 6809.

In the beginning of 1992, Tandy Color Computer users in the US  found

out  about  these  features. Although there has been limited and sometimes

incorrect information about the new functions  of  the  chip,  I  hope  to

bridge that gap with the information provided here.

Remember  that  this  information is of technical nature and makes no

attempt to teach assembly language programming. It  is  ONLY  a  technical

reference  guide for those who already know assembly and wish to use these

features in their programs. Although all of the opcodes for the  6309/6809

chip  are listed in the appendix, only the additional features supplied by

the 6309 will be discussed.

Summary of Features

More registers:

one 8/16 bit 'zero' register

Two 8bit accumulators.

One 16bit concatenated register

One 16bit value register.

One 8bit mode/error register.

One 32bit concatenated register

Two modes: MC68B09E emulation mode and HD63B09EP native mode.

Reduced execution cycles when running in native mode.

Many additional instructions.

Error trapping of illegal instructions and zero divisions.

HD63B09EP Technical Reference Guide                                  Page  2

Description of Additional Registers

The  6309  has  7  additional  registers.  Only 4 of these are actual

registers.  2 are combinations of registers, and the last is a

constant-value register. These registers are:

ACCE  - 8 bit accumulator.

ACCF  - 8 bit accumulator.

W     - 16 bit concatenated register (ACCE and ACCF combined).

V     - 16 bit register (which can only be accessed with the

inter-register instructions).

0     - zero register

MD    - 8 bit mode/error register.

Q     - 32 bit concatenated register (ACCA, ACCB ,ACCE and ACCF

combined).

ACCE  and  ACCF  both  work  the  same  as  the  ACCA  and  ACCB

accumulators.  This  makes for easier programming in math and data oriented

routines.

The W register  is  like  the  D  register  in  the  6809.  It  is  a

concatenated register containing the values of ACCE and ACCF as one 16 bit

value. ACCE is contained in the high 8 bits and ACCF is contained  in  the

low 8 bits.

The  V  register  is a 16 bit register that can only be accessed with

inter-register instructions such a TFR  and  EXG.  The  contents  of  this

register will not change if the CPU is reset, allowing this register to be

used as a constant value for the program.

The 0 register is always zero, independant of reads/writes to it.

It enables a zero value to be used in inter-register operations without

accessing memory, or changing the value of another register.  If

a 0 byte is stored at address $0000, it may also be used to clear large

amounts of memory quickly via 'TFM 0,r+'

The MD register is a mode and error register and works  much  in  the

same way as the CC register. The bit definitions are as follows:

Write bits

Bit 0   -  Execution mode of the 6309.

If clear ( 0 ), the cpu is in 6809 emulation mode.

If set ( 1 ), the cpu is in 6309 native mode.

Bit 1   -  FIRQ mode

If clear ( 0 ), the FIRQ will occur normally.

If set ( 1 ) , the FIRQ will operate the same as the

IRQ

Bits 2 to 5 are unused

Read bits - One of these bits is set when the 6309 traps an error

Bit 6   -  This bit is set ( 1 ) if an illegal instruction is

encountered

Bit 7   -  This bit is set ( 1 ) if a zero division occurs.

The  Q  register  is a 32 bit concatenated register. This register is

the same as the D and W register except for one respect. It  contains  the

values  of  ACCA,  ACCB, ACCE and ACCF respectively. This register is used

mostly with the additional math instructions supplied with the 6309  which

will be discussed later.

HD63B09EP Technical Reference Guide                                  Page  3

Modes of Operation

The 6309 has  two modes of operation; 6809 Emulation  mode  in  which

the  chip  acts  and  executes instructions the same as the 6809, and 6309

Native mode which stores an extra two bytes on the stack when an interrupt

(IRQ) occurs, and executes instructions in fewer clock cycles.

When  in  native  mode, the W register (2 additional bytes) is stored

(PSHS) on the system stack when an interrupt occurs,  it is stored  on  the

stack  right  after  the  D  (general data)  register. Since  ALL register

values are stored on the system stack when an IRQ (NOT  FIRQ  -  See  FIRQ

modes  for  more  information)   occurs,  great  care should be taken when

writing or patching those routines to run in native mode.

Pull <- CC,A,B,E*,F*,DP,Xhi,Xlo,Yhi,Ylo,Uhi,Ulo,PChi,PClo <- Push

* indicates the additional registers stored on the system stack

When in native mode those interrupt routines which modify the  return

address  by modifying  the  10th and 11th byte offsets from the stack (STX

10,S or STY 10,S etc.) will have to be changed to modify the 12th and 13th

byte offsets from the stack (STX 12,S or STY 12,S etc.). If those routines

are not patched to run in native mode they will  either  get  stuck  in  a

continuous loop or will crash the system due to the fact that they are not

returning to the correct address. This poses  a  MAJOR  problem  for  OS-9

Level  II  since  its main interrupt handling routine relies highly on the

changing of the return (PC) address on  the  stack.  Disk  read/write  and

formatting  routines  also  rely  heavily  on  changing  the return address

during an NMI (Non-Maskable Interrupt).

To patch those routines which  do  modify  the  return  address,  the

program  or  routine  must  be disassembled or modified with a disk sector

editing program. Look for instructions such as STX 10,S or STY  10,S  that

has  an  RTI (Return from Interrupt) instruction within the next few lines

of the routine. The line containing STX 10,S or STY 10,S should be changed

to STX 12,S or STY 12,S respectively.

Remember, after those routines are patched, those programs using them

will NOT work in emulation mode and will require native mode to be enabled

upon startup.

Native Mode and Timing Loops

There  is at least one more problem that needs to be addressed. Those

are routines which are dependant on timing loops for accuarate  operation.

Since  the  6309  executes  instructions faster when in native mode, those

routines that use timing loops would be effected. Since this  can  pose  a

problem  and can create erratic operation, the delay value or routine will

need to be changed for the routine to operate correctly.

Those  routines  are  usually   serial-printer   routines,   cassette

read/write  timimg  routines,  software  clocks  and  some disk read/write

routines.

HD63B09EP Technical Reference Guide                                  Page  4

Modes of the Fast Interrupt Request (FIRQ)

The   designers   of  the  6309  decided  that  with  the  additional

instructions and native mode of operation, the FIRQ may be used more  than

it  usually  is.  With  this in mind they decided to allow you to make the

FIRQ run the same as the IRQ and store (PSHS) all the  current  values  of

the  registers  on the system stack. Normally, the FIRQ only stores the CC

(condition code) and the PC (Program Counter/return address) on the stack,

so  to  keep compatability with the 6809, they included it as a selectable

feature in the MD (Mode/status) register.

Inter-Register Instructions

The new Inter-Register instructions (ADCR,  ADDR,  CMPR,  EORR,  ORR,

SBCR,  and  SUBR)  all work the same as their register/memory (ADCA, ADDA,

etc.) counterparts except that they operate between registers. All of  the

new  instructions  use  the  same  post-byte information as the normal TFR

instruction and use the  format  of  R0,R1  (register  0  and  Register  1

respectively)  with  the  result  going  into  R1. See Block Transfers for

information on the TFR block move instructions.

Mixed-size inter-register operations default to using

identical sized register.  So TFR A,X actually executes as TFR D,X.

You could also do 'lea(d) d,pc' type things by doing 'addr pc,d'.  As

the new inter-register instructions can now perform math using the PC

register, REALLY odd possibilities exist.  Try looking at code like

'eorr d,pc', and figuring out where it ends up.

Inter-register instructions with 16-bit r1 and CC or DP (8-bit r2)

are legal, but the results are unknown.

Bit Manipulation of Memory Locations

The  AIM,  EIM,  OIM  and  TIM  instructions  all  do   logical   bit

manipulations  to  locations  in  memory,  with the result stored into the

location, and the respective bits for  each  instruction  set  in  the  CC

register.  They  can be  used in the DIRECT, INDEXED or EXTENDED adressing

modes.

Instruction descriptions:

AIM - AND IN MEMORY

EIM - EOR IN MEMORY

OIM - OR IN MEMORY

TIM - TEST bits IN MEMORY

Instruction format: X, post byte, operand

Where X is the instruction op-code, post-byte contains  the  bits  to

AND,  OR,  EOR or TEST against the memory location, and the operand is the

memory location or indexing post-byte depending on the mode of operation.

Mnemonic format:

Instruction logical operation value, memory location or index operation

Mnemonic example:

AIM #$0F,$E00

The example takes the contents of memory location $E00,  does  a  LOGICAL

and with the Value #$0F and then stores the result back into $E00.

HD63B09EP Technical Reference Guide                                  Page  5

Bit Transfers Between Memory Locations and Registers

The  BAND, BIAND, BOR, BIOR,  BEOR,  BIEOR,  LDBT,  and  STBT  all do

logical  operations  to bits for the n-th bit in a memory location and the

m-th bit of a register. The  LDBT  and  STBT  instructions  allow  you  to

transfer   certain  bits  between  registers  and  memory  locations.  All

instructions allow you  to  specify  which  register  to  use,  which  bit

location  to  use in the register, which bit location to use in the memory

location, and the memory location to use. This allows you  to  transfer/or

do a logical operation with the 7th bit of a register and the 3rd bit of a

memory location. All bits are accessible on either the register or  memory

locations. The only limitations are that the instructions can only be used

with the A and B accumulators and the CC (condition  Code)  registers.  It

should  also  be  noted  that  these  instructions can only be used in the

DIRECT addressing mode.

Instruction description:

BAND  - AND a bit in a register with bit from memory location

BIAND - AND a bit in a register with the complement of the bit in memory

BOR   - OR a bit in a register with a bit from a memory location

BIOR  - OR a bit in a register with the complement of the bit in memory

BEOR  - EOR a bit in a register with a bit from a memory location

BIEOR - EOR a bit in a register with the complement of the bit in memory

LDBT  - Load a bit from a memory location into a bit in a register

STBT  - Store a bit from a register into a memory location.

Instruction format:

x, post-byte, memory location

Where X is  the  instruction  op-code,  the  post-byte  contains  the

register,  source  and destination bit information and the memory location

is the 8 bit value of the memory location to be used (Remember only DIRECT

mode is allowed with these instructions).

Mnemonic format:

instruction, register, source bit, destination bit, memory location

Mnemonic example:

BOR A,1,7,$00

The  example would take the first (1) bit of register A (A) and OR it

into the 7th (7) bit of memory location $00 ($00) of the direct  page  (DP

register value)

The post-byte of these instructions are not the same as the post-byte

used in any other operation (indexed or  inter-register)  as  all  of  the

information  (register,  source  and  destination bit) is contained in one

post-byte value.

HD63B09EP Technical Reference Guide                                  Page  6

Block Transfers

Block  transfers  are used to move a certain number of bytes from one

place in memory to another with the use of one  instruction.  Two  16  bit

registers  (X,  Y,  U or S) are used to specify the source and destination

addresses, and the size of the block to be transferred is  specified  with

the  W register. It should be noted that even though the IRQ and FIRQ only

occur after  the current  instruction  is  finished,  block  moves  can be

interrupted.  After the interrupt returns, the last byte read is read once

more.  i.e. It is read _twice_ by the CPU  This can  cause  problems  with

memory  mapped  I/O devices, so caution  is advised  when using  the block

transfers. There isn't much control over these 4 instructions so the  only

thing applicable for them would be large block moves such as scrolling the

screen or clearing an area in memory with a certain value.

TFM r0+,r1 and TFM r0,r1+ can be considered a poor mans DMA  channel.

Since all the data is either copied into or read from one memory location.

Four types of block transfers have been provided.

Mnemonic examples:

(R0 - source address register, R1 - destination address register.)

TFM r0+,r1+

- Transfer from R0 to R1 in incrementing order.

TFM r0-,r1-

- Transfer from R0 to R1 in decrementing order.

TFM r0+,r1

- Pour from R0 into R1, only incrementing R0 (R1 stays the same).

TFM r0,r1+

- Read from R0 into R1, only incrementing R0 (R1 stays the same).

Mnemonic example:

LDW #$100

LDX #$600

LDY #$700

TFM X+,Y+

The example would move 256 (LDW $100) bytes from #$600 (LDX #$&00) in

memory to #$700 (LDY #$700) in memory,  incrementing  the  value  of  each

register (X and Y), and decrementing the value of the W register each time

a byte if moved.

When moves like this are done, the pointer registers (X and Y in  the

example)  will  not  be  the  same value they were before the transfer was

initiated, but will but will be their original values PLUS  the  value  of

the  W register (#$100 in the example). So in the example once the move is

complete, the value of X will be returned as #$700 and the value of Y will

be returned as #$800. The value of W register will be 0.

The 0 register may be used as a source or destination address register,

and the data will be read from, or written to, address $0000.

It is illegal to use any of the CC, DP, W, V, or PC registers as either

a source or destination register.

HD63B09EP Technical Reference Guide                                  Page  7

New math commands

The 6309 has 3 additional math instructions.  A  16  bit  by  16  bit

signed  multiply  (MULD),  a 16 bit by 8 bit signed divide (DIVD) and a 32

bit by 16 bit signed divide (DIVQ). These instructions can all be used  in

Immediate, direct, indexed and extended addressing modes.

The MULD (16 bit by 16 bit) instruction does a signed multiply of the

contents of the D register and a value from memory (or  in  direct  mode).

The signed result is stored in the Q register.

The  DIVD  (16  bit by 8 bit) instruction does a signed divide of the

contents of the D register with a value from memory (or in  direct  mode).

The  signed  result  is  stored  with  the  quotient  in  W and the modulo

(remainder) in D.

The DIVQ (32 bit by 16 bit) instruction does a signed divide  of  the

contents  of  the Q register with a value from memory (or in direct mode).

The signed result is  stored  with  the  quotient  in  W  and  the  modulo

(remainder) in D.

Error Trapping

The  6309  has an internal error trapping handler that will jump to a

specific location in memory when either an error  is  encountered  in  the

DIVision  instructions  (only divide by zero) or an illegal instruction is

encountered.  When an  error is  encountered, the  6309  will  jump to the

memory  location  contained  in  $FFF0 (and $FFF1)  which  was  originally

reserved by the 6809.

The trap may cause  problems with machines that  have $FF00 hardcoded

with  the  values $0000.  A new EPROM should  be burned to correct for the

new behaviour of the 6309.

As many people  know, an illegal instruction trap  is extremely useful

for debugging  programs,  as it prevents  the entire machine  from crashing

when a bug is encountered.

Note that many pseudo-legal instructions on the 6809 are now illegal

on the 6309, e.g. $1020xxxx executes as an LBRA on a 6809, but results in

a trap on a 6309.

Additional Instructions

The  6309  has  MANY  new  instructions.  Most  are variations of old

instructions of  the  6809  for  use  with  the  new  registers.  The  new

instruction  set  can be used in both native and emulation mode. Here is a

list of the new instructions of the 6309:

ADCD

- Adds immediate or memory operand to the D register plus the current

status of the carry with the result going to D.

ADCR

- Adds two registers together plus the current status of the carry.

HD63B09EP Technical Reference Guide                                  Page  8

ADDE , ADDF, ADDW

- Add of immediate or memory operand to E, F or W with results going

to E, F or W

ADDR

- Adds two registers together

ANDD

- Logical AND of immediate or memory operand to D register with

result going to D.

ANDR

- Logical AND of a register with the contents of another register

ASLD  (Same as LSLD)

- Arithmetic shift left. Shifts D one bit left, clearing LSB.

ASRD

- Arithmetic shift right of the D register with sign extending.

BITD

- Test any bit or bits of the D register.

BITMD

- Test any bit or bits of the MD (mode) register.

CLRD, CLRE, CLRF, CLRW

- Clear register D, E, F or W to zero.

CMPE, CMPF, CMPW

- Compares the contents of E, F or W with the immediate or memory

operand. Sets all CC except H on result.

CMPR

- Compares one register to another and sets all CC bits except H on

result.

COMD, COME, COMF, COMW

- One's complement D ,E, F, or W. Changes all zero's to one's and

all one's to zero's.

DECD, DECE, DECF, DECW

- Decrement D, E, F, or W by 1.

DIVD, DIVQ

- Does a 16 bit by 8 bit (DIVD) or a 32 bit by 16 bit (DIVQ) signed

divide with immediate or memory operand with quotient in W and modulo

(remainder) in D.

EORD

- Logical exclusive OR of D and immediate or memory operand.

EORR

- Logical exclusive OR of one register with the value of another

register.

INCD, INCE, INCF, INCW

- Increment D, E, F or W by 1.

LDE, LDF, LDQ, LDW, LDMD

- Standard loading of E, F, Q, W or MD with immediate data value or

operand from memory. (LDMD only valid with IMMEDIATE mode)

HD63B09EP Technical Reference Guide                                  Page  9

LSLD  (Same as ASLD)

- Logical shift left. Shifts D one bit left, clearing LSB.

LSRD, LSRW

- Logical shift right. Shifts D or W one bit right, clearing MSB.

MULD

- Performs as 16bit by 16bit signed multiply with immediate or operand

from memory. Result stored in Q.

NEGD

- Two's complement D register.

ORD

- Logical OR of register D and immediate or memory operand.

ORR

- Logical OR of one register with another.

PSHSW, PSHUW

- Stores contents of the W register on the (system or user) stack.

PULSW, PULUW

- Pull value from (system or user) stack into register W.

ROLD, ROLW

- Rotate D or W one bit left through the Carry Condition code.

RORD, RORW

- Rotate D or W one bit right through the Carry Condition code.

SBCD

- Subtract an immediate or memory operand plus any borrow in Carry from

contents of D. Result stored in D.

SBCR

- Subtract the value of one register from another plus any borrow in

the CC carry.

SEXW

- sign exdend the W register into the D register.

STE, STF, STQ, STW

- Store register E, F, Q or W to memory location (E,F), two memory

locations(W), or four memory locations (Q).

SUBE, SUBF, SUBW

- Subtract immediate or memory operand from E, F or W. Result stored

back in same register.

SUBR

- Subtract the value of one register from another.

TFM (Block transfer)

- Transfer W number of bytes from one location to another. Returns

pointer registers offset of the starting value in the W register and

returns the W register as 0. Indexed operation only

TSTD, TSTE, TSTF, TSTW

- Test contents of D, E, F or W by setting N and X condition codes

based on data in register.

Opcode and Mnemonics Reference Table                                 Page 10

The Opcode and Mnemonics opcode reference tables  are  both  complete

listings  that  contain both the Opcode instruction and the HEX equivalant

in  all  available  addressing  modes.  The  first   table   is   arranged

sequentially  by  the  binary  opcodes, while the second table is arranged

alphabetically by the Mnemonic instructions.

At the end of the second  table  there are   data  tables  containing

information    on    Bit   transfer/manipulation,   branch   instructions,

inter-register instructions, and general register and  stack  information.

These  are  all  helpful  to the serious assembly language programmer, who

should always have one.

Opcode table

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic       Mode          Cycles       Length  |

| (* 6309)                                6809 (6309)            |

|----------------------------------------------------------------|

|    00          NEG        Direct          6  (5)         2     |

|  * 01          OIM        Direct          6              3     |

|  * 02          AIM        Direct          6              3     |

|    03          COM        Direct          6  (5)         2     |

|    04          LSR        Direct          6  (5)         2     |

|  * 05          EIM        Direct          6              3     |

|    06          ROR        Direct          6  (5)         2     |

|    07          ASR        Direct          6  (5)         2     |

|    08          ASL/LSL    Direct          6  (5)         2     |

|    09          ROL        Direct          6  (5)         2     |

|    0A          DEC        Direct          6  (5)         2     |

|  * 0B          TIM        Direct          6                    |

|    0C          INC        Direct          6  (5)         2     |

|    0D          TST        Direct          6  (4)         2     |

|    0E          JMP        Direct          3  (2)         2     |

|    0F          CLR        Direct          6  (5)         2     |

|    10          (PREBYTE)                                       |

|    11          (PREBYTE)                                       |

|    12          NOP        Inherent        2  (1)         1     |

|    13          SYNC       Inherent        2  (1)         1     |

|  * 14          SEXW       Inherent        4              1     |

|    16          LBRA       Relative        5  (4)         3     |

|    17          LBSR       Relative        9  (7)         3     |

|    19          DAA        Inherent        2  (1)         1     |

|    1A          ORCC       Immediate       3  (2)         2     |

|    1C          ANDCC      Immediate       3              2     |

|    1D          SEX        Inherent        2  (1)         1     |

|    1E          EXG        Immediate       8  (5)         2     |

|    1F          TFR        Immediate       6  (4)         2     |

|    20          BRA        Relative        3              2     |

|    21          BRN        Relative        3              2     |

|    22          BHI        Relative        3              2     |

|    23          BLS        Relative        3              2     |

|    24          BHS/BCC    Relative        3              2     |

|    25          BLO/BCS    Relative        3              2     |

|    26          BNE        Relative        3              2     |

|    27          BEQ        Relative        3              2     |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 11

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|    28          BVC        Relative        3              2     |

|    29          BVS        Relative        3              2     |

|    2A          BPL        Relative        3              2     |

|    2B          BMI        Relative        3              2     |

|    2C          BGE        Relative        3              2     |

|    2D          BLT        Relative        3              2     |

|    2E          BGT        Relative        3              2     |

|    2F          BLE        Relative        3              2     |

|    30          LEAX       Indexed         4+             2     |

|    31          LEAY       Indexed         4+             2     |

|    32          LEAS       Indexed         4+             2     |

|    33          LEAU       Indexed         4+             2     |

|    34          PSHS       Immediate       5+ (4+)        2     |

|    35          PULS       Immediate       5+ (4+)        2     |

|    36          PSHU       Immediate       5+ (4+)        2     |

|    37          PULU       Immediate       5+ (4+)        2     |

|    39          RTS        Inherent        5  (1)         1     |

|    3A          ABX        Inherent        3  (1)         1     |

|    3B          RTI        Inherent      6/15 (17)        1     |

|    3C          CWAI       Immediate      22  (20)        2     |

|    3D          MUL        Inherent       11  (10)        1     |

|    3F          SWI        Inherent       19  (21)        1     |

|    40          NEGA       Inherent        2  (1)         1     |

|    43          COMA       Inherent        2  (1)         1     |

|    44          LSRA       Inherent        2  (1)         1     |

|    46          RORA       Inherent        2  (1)         1     |

|    47          ASRA       Inherent        2  (1)         1     |

|    48          ASLA/LSLA  Inherent        2  (1)         1     |

|    49          ROLA       Inherent        2  (1)         1     |

|    4A          DECA       Inherent        2  (1)         1     |

|    4C          INCA       Inherent        2  (1)         1     |

|    4D          TSTA       Inherent        2  (1)         1     |

|    4F          CLRA       Inherent        2  (1)         1     |

|    50          NEGB       Inherent        2  (1)         1     |

|    53          COMB       Inherent        2  (1)         1     |

|    54          LSRB       Inherent        2  (1)         1     |

|    56          RORB       Inherent        2  (1)         1     |

|    57          ASRB       Inherent        2  (1)         1     |

|    58          ASLB/LSLB  Inherent        2  (1)         1     |

|    59          ROLB       Inherent        2  (1)         1     |

|    5A          DECB       Inherent        2  (1)         1     |

|    5C          INCB       Inherent        2  (1)         1     |

|    5D          TSTB       Inherent        2  (1)         1     |

|    5F          CLRB       Inherent        2  (1)         1     |

|    60          NEG        Indexed         6+             2+    |

|  * 61          OIM        Indexed         6+             3+    |

|  * 62          AIM        Indexed         7              3+    |

|    63          COM        Indexed         6+             2+    |

|    64          LSR        Indexed         6+             2+    |

|  * 65          EIM        Indexed         7+             3+    |

|    66          ROR        Indexed         6+             2+    |

|    67          ASR        Indexed         6+             2+    |

|    68          ASL/LSL    Indexed         6+             2+    |

|    69          ROL        Indexed         6+             2+    |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 12

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|    6A          DEC        Indexed         6+             2+    |

|  * 6B          TIM        Indexed         7+             3+    |

|    6C          INC        Indexed         6+             2+    |

|    6D          TST        Indexed         6+ (5+)        2+    |

|    6E          JMP        Indexed         3+             2+    |

|    6F          CLR        Indexed         6+             2+    |

|    70          NEG        Extended        7  (6)         3     |

|  * 71          OIM        Extended        7              4     |

|  * 72          AIM        Extended        7              4     |

|    73          COM        Extended        7  (6)         3     |

|    74          LSR        Extended        7  (6)         3     |

|    76          ROR        Extended        7  (6)         3     |

|  * 75          EIM        Extended        7              4     |

|    77          ASR        Extended        7  (6)         3     |

|    78          ASL/LSL    Extended        7  (6)         3     |

|    79          ROL        Extended        7  (6)         3     |

|    7A          DEC        Extended        7  (6)         3     |

|  * 7B          TIM        Extended        7              4     |

|    7C          INC        Extended        7  (6)         3     |

|    7D          TST        Extended        7  (5)         3     |

|    7E          JMP        Extended        4  (3)         3     |

|    7F          CLR        Extended        7  (6)         3     |

|    80          SUBA       Immediate       2              2     |

|    81          CMPA       Immediate       2              2     |

|    82          SBCA       Immediate       2              2     |

|    83          SUBD       Immediate       4  (3)         3     |

|    84          ANDA       Immediate       2              2     |

|    85          BITA       Immediate       2              2     |

|    86          LDA        Immediate       2              2     |

|    88          EORA       Immediate       2              2     |

|    89          ADCA       Immediate       2              2     |

|    8A          ORA        Immediate       2              2     |

|    8B          ADDA       Immediate       2              2     |

|    8C          CMPX       Immediate       4  (3)         3     |

|    8D          BSR        Relative        7  (6)         2     |

|    8E          LDX        Immediate       3              3     |

|    90          SUBA       Direct          4  (3)         2     |

|    91          CMPA       Direct          4  (3)         2     |

|    92          SBCA       Direct          4  (3)         2     |

|    93          SUBD       Direct          6  (4)         2     |

|    94          ANDA       Direct          4  (3)         2     |

|    95          BITA       Direct          4  (3)         2     |

|    96          LDA        Direct          4  (3)         2     |

|    97          STA        Direct          4  (3)         2     |

|    98          EORA       Direct          4  (3)         2     |

|    99          ADCA       Direct          4  (3)         2     |

|    9A          ORA        Direct          4  (3)         2     |

|    9B          ADDA       Direct          4  (3)         2     |

|    9C          CMPX       Direct          6  (4)         2     |

|    9D          JSR        Direct          7  (6)         2     |

|    9E          LDX        Direct          5  (4)         2     |

|    9F          STX        Direct          5  (4)         2     |

|    A0          SUBA       Indexed         4+             2+    |

|    A1          CMPA       Indexed         4+             2+    |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 13

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|    A2          SBCA       Indexed         4+             2+    |

|    A3          SUBD       Indexed         6+ (5+)        2+    |

|    A4          ANDA       Indexed         4+             2+    |

|    A5          BITA       Indexed         4+             2+    |

|    A6          LDA        Indexed         4+             2+    |

|    A7          STA        Indexed         4+             2+    |

|    A8          EORA       Indexed         4+             2+    |

|    A9          ADCA       Indexed         4+             2+    |

|    AA          ORA        Indexed         4+             2+    |

|    AB          ADDA       Indexed         4+             2+    |

|    AC          CMPX       Indexed         6+ (5+)        2+    |

|    AD          JSR        Indexed         7+ (6+)        2+    |

|    AE          LDX        Indexed         5+             2+    |

|    AF          STX        Indexed         5+             2+    |

|    B0          SUBA       Extended        5  (4)         3     |

|    B1          CMPA       Extended        5  (4)         3     |

|    B2          SBCA       Extended        5  (4)         3     |

|    B3          SUBD       Extended        7  (5)         3     |

|    B4          ANDA       Extended        5  (4)         3     |

|    B5          BITA       Extended        5  (4)         3     |

|    B6          LDA        Extended        5  (4)         3     |

|    B7          STA        Extended        5  (4)         3     |

|    B8          EORA       Extended        5  (4)         3     |

|    B9          ADCA       Extended        5  (4)         3     |

|    BA          ORA        Extended        5  (4)         3     |

|    BB          ADDA       Extended        5  (4)         3     |

|    BC          CMPX       Extended        7  (5)         3     |

|    BD          JSR        Extended        8  (7)         3     |

|    BE          LDX        Extended        6  (5)         3     |

|    BF          STX        Extended        6  (5)         3     |

|    C0          SUBB       Immediate       2              2     |

|    C1          CMPB       Immediate       2              2     |

|    C2          SBCB       Immediate       2              2     |

|    C3          ADDD       Immediate       4  (3)         3     |

|    C4          ANDB       Immediate       2              2     |

|    C5          BITB       Immediate       2              2     |

|    C6          LDB        Immediate       2              2     |

|    C8          EORB       Immediate       2              2     |

|    C9          ADCB       Immediate       2              2     |

|    CA          ORB        Immediate       2              2     |

|    CB          ADDB       Immediate       2              2     |

|    CC          LDD        Immediate       3              3     |

|  * CD          LDQ        Immediate       5              5     |

|    CE          LDU        Immediate       3              3     |

|    D0          SUBB       Direct          4  (3)         2     |

|    D1          CMPB       Direct          4  (3)         2     |

|    D2          SBCB       Direct          4  (3)         2     |

|    D3          ADDD       Direct          6  (4)         2     |

|    D4          ANDB       Direct          4  (3)         2     |

|    D5          BITB       Direct          4  (3)         2     |

|    D6          LDB        Direct          4  (3)         2     |

|    D7          STB        Direct          4  (3)         2     |

|    D8          EORB       Direct          4  (3)         2     |

|    D9          ADCB       Direct          4  (3)         2     |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 14

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|    DA          ORB        Direct          4  (3)         2     |

|    DB          ADDB       Direct          4  (3)         2     |

|    DC          LDD        Direct          5  (4)         2     |

|    DD          STD        Direct          5  (4)         2     |

|    DE          LDU        Direct          5  (4)         2     |

|    DF          STU        Direct          5  (4)         2     |

|    E0          SUBB       Indexed         4+             2+    |

|    E1          CMPB       Indexed         4+             2+    |

|    E2          SBCB       Indexed         4+             2+    |

|    E3          ADDD       Indexed         6+ (5+)        2+    |

|    E4          ANDB       Indexed         4+             2+    |

|    E5          BITB       Indexed         4+             2+    |

|    E6          LDB        Indexed         4+             2+    |

|    E7          STB        Indexed         4+             2+    |

|    E8          EORB       Indexed         4+             2+    |

|    E9          ADCB       Indexed         4+             2+    |

|    EA          ORB        Indexed         4+             2+    |

|    EB          ADDB       Indexed         4+             2+    |

|    EC          LDD        Indexed         5+             2+    |

|    ED          STD        Indexed         5+             2+    |

|    EE          LDU        Indexed         5+             2+    |

|    EF          STU        Indexed         5+             2+    |

|    F0          SUBB       Extended        5  (4)         3     |

|    F1          CMPB       Extended        5  (4)         3     |

|    F2          SBCB       Extended        5  (4)         3     |

|    F3          ADDD       Extended        7  (5)         3     |

|    F4          ANDB       Extended        5  (4)         3     |

|    F5          BITB       Extended        5  (4)         3     |

|    F6          LDB        Extended        5  (4)         3     |

|    F7          STB        Extended        5  (4)         3     |

|    F8          EORB       Extended        5  (4)         3     |

|    F9          ADCB       Extended        5  (4)         3     |

|    FA          ORB        Extended        5  (4)         3     |

|    FB          ADDB       Extended        5  (4)         3     |

|    FC          LDD        Extended        6  (5)         3     |

|    FD          STD        Extended        6  (5)         3     |

|    FE          LDU        Extended        6  (5)         3     |

|    FF          STU        Extended        6  (5)         3     |

|    1021        LBRN       Reletive       5/6 ( )         4     |

|    1022        LBHI       Reletive       5/6 ( )         4     |

|    1023        LBLS       Reletive       5/6 ( )         4     |

|    1024        LBHS/LBCC  Reletive       5/6 ( )         4     |

|    1025        LBCS/LBLO  Reletive       5/6 ( )         4     |

|    1026        LBNE       Reletive       5/6 ( )         4     |

|    1027        LBEQ       Reletive       5/6 ( )         4     |

|    1028        LBVC       Reletive       5/6 ( )         4     |

|    1029        LBVS       Reletive       5/6 ( )         4     |

|    102A        LBPL       Reletive       5/6 ( )         4     |

|    102B        LBMI       Reletive       5/6 ( )         4     |

|    102C        LBGE       Reletive       5/6 ( )         4     |

|    102D        LBLT       Reletive       5/6 ( )         4     |

|    102E        LBGT       Reletive       5/6 ( )         4     |

|    102F        LBLE       Reletive       5/6 ( )         4     |

|  * 1030        ADDR       Register        4              3     |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 15

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|  * 1031        ADCR       Register        4              3     |

|  * 1032        SUBR       Register        4              3     |

|  * 1033        SBCR       Register        4              3     |

|  * 1034        ANDR       Register        4              3     |

|  * 1035        ORR        Register        4              3     |

|  * 1036        EORR       Register        4              3     |

|  * 1037        CMPR       Register        4              3     |

|  * 1038        PSHSW      Register        6              2     |

|  * 1039        PULSW      Register        6              2     |

|  * 103A        PSHUW      Register        6              2     |

|  * 103B        PULUW      Register        6              2     |

|    103F        SWI2       Inherent        20 (22)        2     |

|  * 1040        NEGD       Inherent        3  (2)         2     |

|  * 1043        COMD       Inherent        3  (2)         2     |

|  * 1044        LSRD       Inherent        3  (2)         2     |

|  * 1046        RORD       Inherent        3  (2)         2     |

|  * 1047        ASRD       Inherent        3  (2)         2     |

|  * 1048        ASLD/LSLD  Inherent        3  (2)         2     |

|  * 1049        ROLD       Inherent        3  (2)         2     |

|  * 104A        DECD       Inherent        3  (2)         2     |

|  * 104C        INCD       Inherent        3  (2)         2     |

|  * 104D        TSTD       Inherent        3  (2)         2     |

|  * 104F        CLRD       Inherent        3  (2)         2     |

|  * 1053        COMW       Inherent        3  (2)         2     |

|  * 1054        LSRW       Inherent        3  (2)         2     |

|  * 1056        RORW       Inherent        3  (2)         2     |

|  * 1059        ROLW       Inherent        3  (2)         2     |

|  * 105A        DECW       Inherent        3  (2)         2     |

|  * 105C        INCW       Inherent        3  (2)         2     |

|  * 105D        TSTW       Inherent        3  (2)         2     |

|  * 105F        CLRW       Inherent        3  (2)         2     |

|  * 1080        SUBW       Immediate       5  (4)         4     |

|  * 1081        CMPW       Immediate       5  (4)         4     |

|  * 1082        SBCD       Immediate       5  (4)         4     |

|    1083        CMPD       Immediate       5  (4)         4     |

|  * 1084        ANDD       Immediate       5  (4)         4     |

|  * 1085        BITD       Immediate       5  (4)         4     |

|  * 1086        LDW        Immediate       5  (4)         4     |

|  * 1088        EORD       Immediate       5  (4)         4     |

|  * 1089        ADCD       Immediate       5  (4)         4     |

|  * 108A        ORD        Immediate       5  (4)         4     |

|  * 108B        ADDW       Immediate       5  (4)         4     |

|    108C        CMPY       Immediate       5  (4)         4     |

|    108E        LDY        Immediate       5  (4)         4     |

|  * 1090        SUBW       Direct          7  (5)         3     |

|  * 1091        CMPW       Direct          7  (5)         3     |

|  * 1092        SBCD       Direct          7  (5)         3     |

|    1093        CMPD       Direct          7  (5)         3     |

|  * 1094        ANDD       Direct          7  (5)         3     |

|  * 1095        BITD       Direct          7  (5)         3     |

|  * 1096        LDW        Direct          6  (5)         3     |

|  * 1097        STW        Direct          6  (5)         3     |

|  * 1098        EORD       Direct          7  (5)         3     |

|  * 1099        ADCD       Direct          7  (5)         3     |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 16

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|  * 109A        ORD        Direct          7  (5)         3     |

|  * 109B        ADDW       Direct          7  (5)         3     |

|    109C        CMPY       Direct          7  (5)         3     |

|    109E        LDY        Direct          6  (5)         3     |

|    109F        STY        Direct          6  (5)         3     |

|  * 10A0        SUBW       Indexed         7+ (6+)        3+    |

|  * 10A1        CMPW       Indexed         7+ (6+)        3+    |

|  * 10A2        SBCD       Indexed         7+ (6+)        3+    |

|    10A3        CMPD       Indexed         7+ (6+)        3+    |

|  * 10A4        ANDD       Indexed         7+ (6+)        3+    |

|  * 10A5        BITD       Indexed         7+ (6+)        3+    |

|  * 10A6        LDW        Indexed         6+             3+    |

|  * 10A7        STW        Indexed         6+             3+    |

|  * 10A8        EORD       Indexed         7+ (6+)        3+    |

|  * 10A9        ADCD       Indexed         7+ (6+)        3+    |

|  * 10AA        ORD        Indexed         7+ (6+)        3+    |

|  * 10AB        ADDW       Indexed         7+ (6+)        3+    |

|    10AC        CMPY       Indexed         7+ (6+)        3+    |

|    10AE        LDY        Indexed         6              3+    |

|    10AF        STY        Indexed         6              3+    |

|  * 10B0        SUBW       Extended        8  (6)         4     |

|  * 10B1        CMPW       Extended        8  (6)         4     |

|  * 10B2        SBCD       Extended        8  (6)         4     |

|    10B3        CMPD       Extended        8  (6)         4     |

|  * 10B4        ANDD       Extended        8  (6)         4     |

|  * 10B5        BITD       Extended        8  (6)         4     |

|  * 10B6        LDW        Extended        7  (6)         4     |

|  * 10B7        STW        Extended        7  (6)         4     |

|  * 10B8        EORD       Extended        8  (6)         4     |

|  * 10B9        ADCD       Extended        8  (6)         4     |

|  * 10BA        ORD        Extended        8  (6)         4     |

|  * 10BB        ADDW       Extended        8  (6)         4     |

|    10BC        CMPY       Extended        8  (6)         4     |

|    10BE        LDY        Extended        7  (6)         4     |

|    10BF        STY        Extended        7  (6)         4     |

|    10CE        LDS        Immediate       4              4     |

|  * 10DC        LDQ        Direct          8  (7)         3     |

|  * 10DD        STQ        Direct          8  (7)         3     |

|    10DE        LDS        Direct          6  (5)         3     |

|    10DF        STS        Direct          6  (5)         3     |

|  * 10DC        LDQ        Indexed         8+             3+    |

|  * 10DD        STQ        Indexed         8+             3+    |

|    10EE        LDS        Indexed         6+             3+    |

|    10EF        STS        Indexed         6+             3+    |

|  * 10DC        LDQ        Extended        9  (8)         4     |

|  * 10DD        STQ        Extended        9  (8)         4     |

|    10FE        LDS        Extended        7  (6)         4     |

|    10FF        STS        Extended        7  (6)         4     |

|  * 1130        BAND       Memory          7  (6)         4     |

|  * 1131        BIAND      Memory          7  (6)         4     |

|  * 1132        BOR        Memory          7  (6)         4     |

|  * 1133        BIOR       Memory          7  (6)         4     |

|  * 1134        BEOR       Memory          7  (6)         4     |

|  * 1135        BIEOR      Memory          7  (6)         4     |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 17

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|  * 1136        LDBT       Memory          7  (6)         4     |

|  * 1137        STBT       Memory          8  (7)         4     |

|  * 1138        TFM R+,R+  Register        6+3n           3     |

|  * 1139        TFM R-,R-  Register        6+3n           3     |

|  * 113A        TFM R+,R   Register        6+3n           3     |

|  * 113B        TFM R,R+   Register        6+3n           3     |

|  * 113C        BITMD      Immediate       4              3     |

|  * 113D        LDMD       Immediate       5              5     |

|    113F        SWI2       Inherent        20 ( )         2     |

|  * 1143        COME       Inherent        3  (2)         2     |

|  * 114A        DECE       Inherent        3  (2)         2     |

|  * 114C        INCE       Inherent        3  (2)         2     |

|  * 114D        TSTE       Inherent        3  (2)         2     |

|  * 114F        CLRE       Inherent        3  (2)         2     |

|  * 1153        COMF       Inherent        3  (2)         2     |

|  * 115A        DECF       Inherent        3  (2)         2     |

|  * 115C        INCF       Inherent        3  (2)         2     |

|  * 115D        TSTF       Inherent        3  (2)         2     |

|  * 115F        CLRF       Inherent        3  (2)         2     |

|    11AC        CMPS       Indexed         7  ( )         3     |

|  * 1180        SUBE       Immediate       3              3     |

|  * 1181        CMPE       Immediate       3              3     |

|    1183        CMPU       Immediate       5  (4)         4     |

|  * 1186        LDE        Immediate       3              3     |

|  * 118B        ADDE       Immediate       3              3     |

|    118C        CMPS       Immediate       5  (4)         4     |

|  * 118D        DIVD       Immediate       25             4     |

|  * 118E        DIVQ       Immediate       36             4     |

|  * 118F        MULD       Immediate       28             4     |

|  * 1190        SUBE       Direct          5  (4)         3     |

|  * 1191        CMPE       Direct          5  (4)         3     |

|    1193        CMPU       Direct          7  (5)         3     |

|  * 1196        LDE        Direct          5  (4)         3     |

|  * 1197        STE        Direct          5  (4)         3     |

|  * 119B        ADDE       Direct          5  (4)         3     |

|    119C        CMPS       Direct          7  (5)         3     |

|  * 119D        DIVD       Direct          27 (26)        3     |

|  * 119E        DIVQ       Direct          36 (35)        3     |

|  * 119F        MULD       Direct          30 (29)        3     |

|  * 11A0        SUBE       Indexed         5+             3+    |

|  * 11A1        CMPE       Indexed         5+             3+    |

|    11A3        CMPU       Indexed         7+ (6+)        3+    |

|  * 11A6        LDE        Indexed         5+             3+    |

|  * 11A7        STE        Indexed         5+             3+    |

|  * 11AB        ADDE       Indexed         5+             3+    |

|    11AC        CMPS       Indexed         7+ (6+)        3+    |

|  * 11AD        DIVD       Indexed         27+            3+    |

|  * 11AE        DIVQ       Indexed         36+            3+    |

|  * 11AF        MULD       Indexed         30+            3+    |

|  * 11B0        SUBE       Extended        6  (5)         4     |

----------------------------------------------------------------

Opcode and Mnemonics opcode reference table                          Page 18

________________________________________________________________

|                                                                |

|  Opcode      Mnemonic        Mode         Cycles       Length  |

| (* 6309)                                                       |

|----------------------------------------------------------------|

|  * 11B1        CMPE       Extended        6  (5)         4     |

|    11B3        CMPU       Extended        8  (6)         4     |

|  * 11B6        LDE        Extended        6  (5)         4     |

|  * 11B7        STE        Extended        6  (5)         4     |

|  * 11BB        ADDE       Extended        6  (5)         4     |

|    11BC        CMPS       Extended        8  (6)         4     |

|  * 11BD        DIVD       Extended        28 (27)        4     |

|  * 11BE        DIVQ       Extended        37 (36)        4     |

|  * 11BF        MULD       Extended        31 (30)        4     |

|  * 11C0        SUBF       Immediate       3              3     |

|  * 11C1        CMPF       Immediate       3              3     |

|  * 11C6        LDF        Immediate       3              3     |

|  * 11CB        ADDF       Immediate       3              3     |

|  * 11D0        SUBF       Direct          5  (4)         3     |

|  * 11D1        CMPF       Direct          5  (4)         3     |

|  * 11D6        LDF        Direct          5  (4)         3     |

|  * 11D7        STF        Direct          5  (4)         3     |

|  * 11DB        ADDF       Direct          5  (4)         3     |

|  * 11E0        SUBF       Indexed         5+             3+    |

|  * 11E1        CMPF       Indexed         5+             3+    |

|  * 11E6        LDF        Indexed         5+             3+    |

|  * 11E7        STF        Indexed         5+             3+    |

|  * 11EB        ADDF       Indexed         5+             3+    |

|  * 11F0        SUBF       Extended        6 (5)          4     |

|  * 11F1        CMPF       Extended        6 (5)          4     |

|  * 11F6        LDF        Extended        6 (5)          4     |

|  * 11F7        STF        Extended        6 (5)          4     |

|  * 11FB        ADDF       Extended        6 (5)          4     |

----------------------------------------------------------------

Mnemonics Reference Table                                            Page 19

Mnemonics Table

_________________________________________________________________________

|  Mnem  |   Immed.   |   Direct   |   Indexed  |  Extended  |  Inherent  |

|        |            |            |            |            |            |

|        | OP  ~/~  # | OP  ~/~  + | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # |

|--------+------------+------------+------------+------------+------------|

| ABX    |            |            |            |            | 3A  3/1  1 |

| ADCA   | 89   2   2 | 99  4/3  2 | A9  4+   2+| B9  5/4  3 |            |

| ADCB   | C9   2   2 | D9  4/3  2 | E9  4+   2+| F9  5/3  3 |            |

|*ADCD   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 89         | 99         | A9         | B9         |            |

|--------+------------+------------+------------+------------+------------|

| ADDA   | 8B   2   2 | 9B  4/3  2 | AB  4+   2+| BB  5/4  3 |            |

| ADDB   | CB   2   2 | DB  4/3  2 | EB  4+   2+| FB  5/4  3 |            |

| ADDD   | C3  4/3  3 | D3  6/4  2 | E3 6+/5+ 2+| F3  7/5  3 |            |

|*ADDE   | 11   3   3 | 11  5/4  3 | 11  5+   3+| 11  6/5  4 |            |

|        | 8B         | 9B         | AB         | BB         |            |

|*ADDF   | 11   3   3 | 11  5/4  3 | 11  5+   3+| 11  6/5  4 |            |

|        | CB         | DB         | EB         | FB         |            |

|*ADDW   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 8B         | 9B         | AB         | BB         |            |

|--------+------------+------------+------------+------------+------------|

|*AIM    |            | 02   6   3 | 62   7+  3+| 72   7   4 |            |

|--------+------------+------------+------------+------------+------------|

| ANDA   | 84   2   2 | 94  4/3  2 | A4   4+  2 | B4  5/4  3 |            |

| ANDB   | C4   2   2 | D4  4/3  2 | E4   4+  2 | F4  5/4  3 |            |

| ANDCC  | 1C   3   2 |            |            |            |            |

|*ANDD   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 84         | 94         | A4         | B4         |            |

|--------+------------+------------+------------+------------+------------|

| ASLA   |            |            |            |            | 48  2/1  1 |

| ASLB   |            |            |            |            | 58  2/1  1 |

|*ASLD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 48         |

| ASL    |            | 08  6/5  2 | 68  6+   2+| 78  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| ASRA   |            |            |            |            | 47  2/1  1 |

| ASRB   |            |            |            |            | 57  2/1  1 |

|*ASRD   |            |            |            |            | 10  3/2  1 |

|        |            |            |            |            | 47         |

| ASR    |            | 07  6/6  2 | 67  6+   2+| 77  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| BITA   | 85   2   2 | 95  4/3  2 | A5   4+  2+| B5  5/4  3 |            |

| BITB   | C5   2   2 | D5  4/3  2 | E5   4+  2+| F5  5/4  3 |            |

| BITD   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 85         | 95         | A5         | B5         |            |

| BITMD  | 11   4   3 |            |            |            |            |

|        | 3C         |            |            |            |            |

|--------+------------+------------+------------+------------+------------|

| CLRA   |            |            |            |            | 4F  2/1  1 |

| CLRB   |            |            |            |            | 5F  2/1  1 |

|*CLRD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 4F         |

|*CLRE   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 4F         |

|*CLRF   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 5F         |

|*CLRW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 5F         |

| CLR    |            | 0F  6/5  2 | 6F   6+  2+| 7F  7/6  3 |            |

-------------------------------------------------------------------------

Mnemonics Reference Table                                            Page 20

_________________________________________________________________________

|  Mnem  |   Immed.   |   Direct   |   Indexed  |  Extended  |  Inherent  |

|        |            |            |            |            |            |

|        | OP  ~/~  # | OP  ~/~  + | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # |

|--------+------------+------------+------------+------------+------------|

| CMPA   | 81   2   2 | 91  4/3  2 | A1   4+  2+| B1  5/4  3 |            |

| CMPB   | C1   2   2 | D1  4/3  2 | E1   4+  2+| F1  5/4  3 |            |

| CMPD   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 83         | 93         | A3         | B3         |            |

|*CMPE   | 11   3   3 | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        | 81         | 91         | A1         | B1         |            |

|*CMPF   | 11   3   3 | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        | C1         | D1         | E1         | F1         |            |

| CMPS   | 11  5/4  4 | 11  7/5  3 | 11 7+/6+ 3+| 11  8/6  4 |            |

|        | 8C         | 9C         | AC         | BC         |            |

| CMPU   | 11  5/4  4 | 11  7/5  3 | 11 7+/6+ 3+| 11  8/6  4 |            |

|        | 83         | 93         | A3         | B3         |            |

|*CMPW   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 81         | 91         | A1         | B1         |            |

| CMPX   | 8C  4/3  3 | 9C  6/4  2 | AC 6+/5+ 2+| BC  7/5  3 |            |

| CMPY   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 8C         | 9C         | AC         | BC         |            |

|--------+------------+------------+------------+------------+------------|

| COMA   |            |            |            |            | 43  2/1  1 |

| COMB   |            |            |            |            | 53  2/1  1 |

|*COMD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 43         |

|*COME   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 43         |

|*COMF   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 53         |

|*COMW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 53         |

| COM    |            | 03  6/5  2 | 63   6+  2+| 73  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| CWAI   | 3C 22/20 2 |            |            |            |            |

|--------+------------+------------+------------+------------+------------|

| DAA    |            |            |            |            | 19  2/1  1 |

|--------+------------+------------+------------+------------+------------|

| DECA   |            |            |            |            | 4A  2/1  1 |

| DECB   |            |            |            |            | 5A  2/1  1 |

|*DECD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 4A         |

|*DECE   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 4A         |

|*DECF   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 5A         |

|*DECW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 5A         |

| DEC    |            | 0A  6/5  2 | 6A   6+  2+| 7A  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

|*DIVD   | 11   25  3 | 11 27/26 3 | 11  27+  3+| 11 28/27 4 |            |

|        | 8D         | 9D         | AD         | BD         |            |

|*DIVQ   | 11   34  4 | 11 36/35 3 | 11  36+  3+| 11 37/36 4 |            |

|        | 8E         | 9E         | AE         | BE         |            |

|--------+------------+------------+------------+------------+------------|

|*EIM    |            | 05   6   3 | 65   7+  3+| 75   7   4 |            |

|--------+------------+------------+------------+------------+------------|

| EORA   | 88   2   2 | 98  4/3  2 | A8   4+  2+| B8  5/4  3 |            |

| EORB   | C8   2   # | D8  4/3  2 | E8   4+  2+| F8  5/4  3 |            |

|*EORD   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 88         | 98         | A8         | B8         |            |

-------------------------------------------------------------------------

Mnemonics Reference Table                                            Page 21

_________________________________________________________________________

|  Mnem  |   Immed.   |   Direct   |   Indexed  |  Extended  |  Inherent  |

|        |            |            |            |            |            |

|        | OP  ~/~  # | OP  ~/~  + | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # |

|--------+------------+------------+------------+------------+------------|

| EXG    | 1E  8/5  2 |            |            |            |            |

|--------+------------+------------+------------+------------+------------|

| INCA   |            |            |            |            | 4C  2/1  1 |

| INCB   |            |            |            |            | 5C  2/1  1 |

|*INCD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 4C         |

|*INCE   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 4C         |

|*INCF   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 5C         |

|*INCW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 5C         |

| INC    |            | 0C  6/5  2 | 6C   6+  2+| 7C  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| JMP    |            | 0E  3/2  2 | 6E   3+  2+| 7E  4/3  3 |            |

|--------+------------+------------+------------+------------+------------|

| JSR    |            | 9D  7/6  2 | AD 7+/6+ 2+| BD  8/7  3 |            |

|--------+------------+------------+------------+------------+------------|

| LDA    | 86   2   2 | 96  4/3  2 | A6   4+  2+| B6  5/4  3 |            |

| LDB    | C6   2   2 | D6  4/3  2 | E6   4+  2+| F6  5/4  3 |            |

| LDD    | CC   3   3 | DC  5/4  2 | EC   5+  2+| FC  6/5  3 |            |

|*LDE    | 11   3   3 | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        | 86         | 96         | A6         | B6         |            |

|*LDF    | 11   3   3 | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        | C6         | D6         | E6         | F6         |            |

|*LDQ    | CD   5   5 | 10  8/7  3 | 10   8+  3+| 10  9/8  4 |            |

|        |            | DC         | EC         | FC         |            |

| LDS    | 10   4   4 | 10  6/5  3 | 10   6+  3+| 10  7/6  4 |            |

|        | CE         | DE         | EE         | FE         |            |

| LDU    | CE   3   3 | DE  5/4  2 | EE   5+  2+| FE  6/5  3 |            |

|*LDW    | 10   4   4 | 10  6/5  3 | 10   6+  3+| 10  7/6  4 |            |

|        | 86         | 96         | A6         | B6         |            |

| LDX    | 8E   3   3 | 9E  5/4  2 | AE   5+  2+| BE  6/5  3 |            |

| LDY    | 10   4   4 | 10  6/5  3 | 10   6+  3+| 10  7/6  4 |            |

|        | 8E         | 9E         | AE         | BE         |            |

|*LDMD   | 11   5   3 |            |            |            |            |

|        | 3D         |            |            |            |            |

|--------+------------+------------+------------+------------+------------|

| LEAS   |            |            | 32   4+  2+|            |            |

| LEAU   |            |            | 33   4+  2+|            |            |

| LEAX   |            |            | 30   4+  2+|            |            |

| LEAY   |            |            | 31   4+  2+|            |            |

|--------+------------+------------+------------+------------+------------|

| LSLA/LSLB/LSLD/LSL - Same as ASL                                        |

|--------+------------+------------+------------+------------+------------|

| LSRA   |            |            |            |            | 44  2/1  1 |

| LSRB   |            |            |            |            | 54  2/1  1 |

|*LSRD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 44         |

|*LSRW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 54         |

| LSR    |            | 04  6/5  2 | 64   6+  2+| 74  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| MUL    |            |            |            |            | 3D 11/10 1 |

|*MULD   | 11   28  4 | 11 30/29 3 | 11  30+  3+| 11 31/30 4 |            |

|        | 8F         | 9F         | AF         | BF         |            |

-------------------------------------------------------------------------

Mnemonics Reference Table                                            Page 22

_________________________________________________________________________

|  Mnem  |   Immed.   |   Direct   |   Indexed  |  Extended  |  Inherent  |

|        |            |            |            |            |            |

|        | OP  ~/~  # | OP  ~/~  + | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # |

|--------+------------+------------+------------+------------+------------|

| NEGA   |            |            |            |            | 40  2/1  1 |

| NEGB   |            |            |            |            | 50  2/1  1 |

|*NEGD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 40         |

| NEG    |            | 00  6/5  2 | 60   6+  2+| 70  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| NOP    |            |            |            |            | 12  2/1  1 |

|--------+------------+------------+------------+------------+------------|

|*OIM    |            | 01   6   3 | 61   7+  3+| 71   7   4 |            |

|--------+------------+------------+------------+------------+------------|

| ORA    | 8A   2   2 | 9A  4/3  2 | AA   4+  2 | BA  5/4  3 |            |

| ORB    | CA   2   2 | DA  4/3  2 | EA   4+  2 | FA  5/4  3 |            |

| ORCC   | 1A  3/2  2 |            |            |            |            |

|*ORD    | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 8A         | 9A         | AA         | BA         |            |

|--------+------------+------------+------------+------------+------------|

| PSHS   | 34 5+/4+ 2 |            |            |            |            |

| PSHU   | 36 5+/4+ 2 |            |            |            |            |

|*PSHSW  | 10   6   2 |            |            |            |            |

|        | 38   6   2 |            |            |            |            |

|*PSHUW  | 10   6   2 |            |            |            |            |

|        | 3A   6   2 |            |            |            |            |

|--------+------------+------------+------------+------------+------------|

| PULS   | 35 5+/4+ 2 |            |            |            |            |

| PULU   | 37 5+/4+ 2 |            |            |            |            |

|*PULSW  | 10   6   2 |            |            |            |            |

|        | 39         |            |            |            |            |

|*PULUW  | 10   6   2 |            |            |            |            |

|        | 3B         |            |            |            |            |

|--------+------------+------------+------------+------------+------------|

| ROLA   |            |            |            |            | 49  2/1  1 |

| ROLB   |            |            |            |            | 59  2/1  1 |

|*ROLD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 49         |

|*ROLW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 59         |

| ROL    |            | 09  6/5  2 | 69   6+  2+| 79  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| RORA   |            |            |            |            | 46  2/1  1 |

| RORB   |            |            |            |            | 56  2/1  1 |

|*RORD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 46         |

|*RORW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 56         |

| ROR    |            | 06  6/5  2 | 66   6+  2+| 76  7/6  3 |            |

|--------+------------+------------+------------+------------+------------|

| RTI    |            |            |            |            | 3B  6/17 1 |

|        |            |            |            |            |    15/17   |

|--------+------------+------------+------------+------------+------------|

| RTS    |            |            |            |            | 39  5/4  1 |

|--------+------------+------------+------------+------------+------------|

| SBCA   | 82   2   2 | 92  4/3  2 | A2   4+  2+| B2  5/4  3 |            |

| SBCB   | C2   2   2 | D2  4/3  2 | E2   4+  2+| F2  5/2  3 |            |

|*SBCD   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 82         | 92         | A2         | B2         |            |

|--------+------------+------------+------------+------------+------------|

| SEX    |            |            |            |            | 1D  2/1  1 |

|*SEXW   |            |            |            |            | 14   4   1 |

-------------------------------------------------------------------------

Mnemonics Reference Table                                            Page 23

_________________________________________________________________________

|  Mnem  |   Immed.   |   Direct   |   Indexed  |  Extended  |  Inherent  |

|        |            |            |            |            |            |

|        | OP  ~/~  # | OP  ~/~  + | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # |

|--------+------------+------------+------------+------------+------------|

| STA    |            | 97  4/3  2 | A7   4+  2+| B7  5/4  3 |            |

| STB    |            | D7  4/3  2 | E7   4+  2+| F7  5/4  3 |            |

| STD    |            | DD  5/4  2 | ED   5+  2+| FD  6/5  3 |            |

|*STE    |            | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        |            | 97         | A7         | B7         |            |

|*STF    |            | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        |            | D7         | E7         | F7         |            |

|*STQ    |            | 10  8/7  3 | 10   8+  3+| 10  9/8  4 |            |

|        |            | DD         | ED         | FD         |            |

|*STS    |            | 10  6/5  3 | 10   6+  3+| 10  7/6  4 |            |

|        |            | DF         | EF         | FF         |            |

| STU    |            | DF  5/4  2 | EF   5+  2+| FF  6/5  3 |            |

|*STW    |            | 10  6/5  3 | 10   6+  3+| 10  7/6  4 |            |

|        |            | 97         | A7         | B7         |            |

| STX    |            | 9F  5/4  2 | AF   5+  2+| BF  6/5  3 |            |

| STY    |            | 10  6/5  3 | 10   6+  3+| 10  7/6  4 |            |

|        |            | 9F         | AF         | BF         |            |

|--------+------------+------------+------------+------------+------------|

| SUBA   | 80   2   2 | 90  4/3  2 | A0   4+  2+| B0  5/4  3 |            |

| SUBB   | C0   2   2 | D0  4/3  2 | E0   4+  2+| F0  5/4  3 |            |

| SUBD   | 83  4/3  3 | 93  6/4  3 | A3 6+/5+ 2+| B3  7/5  3 |            |

|*SUBE   | 11   3   3 | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        | 80         | 90         | A0         | B0         |            |

|*SUBF   | 11   3   3 | 11  5/4  3 | 11   5+  3+| 11  6/5  4 |            |

|        | C0         | D0         | E0         | F0         |            |

|*SUBW   | 10  5/4  4 | 10  7/5  3 | 10 7+/6+ 3+| 10  8/6  4 |            |

|        | 80         | 90         | A0         | B0         |            |

|--------+------------+------------+------------+------------+------------|

| SWI    |            |            |            |            | 3F 19/21 1 |

| SWI2   |            |            |            |            | 10 20/22 2 |

|        |            |            |            |            | 3F         |

| SWI3   |            |            |            |            | 11 20/22 2 |

|        |            |            |            |            | 3F         |

|--------+------------+------------+------------+------------+------------|

| SYNC   |            |            |            |            | 13 2+/1+ 1 |

|--------+------------+------------+------------+------------+------------|

| TFR   1| 1F  6/4  2 |            |            |            |            |

|--------+------------+------------+------------+------------+------------|

|*TIM    |            | 0B   6   3 | 6B   7+  3+| 7B   5   4 |            |

|--------+------------+------------+------------+------------+------------|

| TSTA   |            |            |            |            | 4D  2/1  1 |

| TSTB   |            |            |            |            | 5D  2/1  1 |

|*TSTD   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 4D         |

|*TSTE   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 4D         |

|*TSTF   |            |            |            |            | 11  3/2  2 |

|        |            |            |            |            | 5D         |

|*TSTW   |            |            |            |            | 10  3/2  2 |

|        |            |            |            |            | 5D         |

| TST    |            | 0D  6/4  2 | 6D 6+/5+ 2+| 7D  7/5  3 |            |

_________________________________________________________________________

Mnemonics Reference Table                                            Page 24

Branch Instructions

_____________________     _____________________     _____________________

|  Mnem  |   Immed.   |   |  Mnem  |   Immed.   |   |  Mnem  |   Immed.   |

|        |            |   |        |            |   |        |            |

|        | OP  ~/~  # |   |        | OP  ~/~  # |   |        | OP  ~/~  # |

|--------+------------+   |--------+------------+   |--------+------------+

| BCC    | 24   3   2 |   | BLE    | 2F   3   2 |   | BPL    | 2A   3   2 |

| LBCC   | 10  5/6  4 |   | LBLE   | 10  5/6  4 |   | LBPL   | 10  5/6  4 |

|        | 24         |   |        | 2F         |   |        | 2A         |

| BCS    | 25   3   2 |   | BLO    | 25   3   2 |   | BRA    | 20   3   2 |

| LBCS   | 10  5/6  4 |   | LBLO   | 10  5/6  4 |   | LBRA   | 16  5/4  3 |

|        | 25         |   |        | 25         |   |        |            |

| BEQ    | 27   3   2 |   | BLS    | 23   3   2 |   | BRN    | 21   3   2 |

| LBEQ   | 10  5/6  4 |   | LBLS   | 10  5/6  4 |   | LBRN   | 10  5/6  4 |

|        | 27         |   |        | 23         |   |        | 21         |

| BGE    | 2C   3   2 |   | BLT    | 2D   3   2 |   | BSR    | 8D  7/6  2 |

| LBGE   | 10  5/6  4 |   | LBLT   | 10  5/6  4 |   | LBSR   | 17  9/7  3 |

|        | 2C         |   |        | 2D         |   |        |            |

| BGT    | 2E   3   2 |   | BMI    | 28   3   2 |   | BVC    | 28   3   2 |

| LBGT   | 10  5/6  4 |   | LBMI   | 10  5/6  4 |   | LBVC   | 10  5/6  4 |

|        | 2E         |   |        | 28         |   |        | 28         |

| BHI    | 22   3   2 |   | BNE    | 26   3   2 |   | BVS    | 29   3   2 |

| LBHI   | 10  5/6  4 |   | LBNE   | 10  5/6  4 |   | LBVS   | 10  5/6  4 |

|        | 22         |   |        | 26         |   |        | 29         |

| BHS    | 2F   3   2 |    ---------------------     ---------------------

| LBHS   | 10  5/6  4 |

|        | 2F         |

---------------------

Bit Transfer/Manipulation

_____________________

|  Mnem  |   Direct   |                  Post-Byte

|        |            |

|        | OP  ~/~  # |          --------------------------

|--------+------------|         | 7  6 | 5  4  3 | 2  1  0 |

|*BAND   | 11  7/6  4 |          --------------------------

|        | 30         |

|*BIAND  | 11  7/6  4 |          Bits 7 and 6: Register

|        | 31         |

|*BOR    | 11  7/6  4 |            00 - CC   10 - B

|        | 32         |            01 - A    11 - Unused

|*BIOR   | 11  7/6  4 |

|        | 33         |          Bits 5, 4 and 3: Source Bit

|*BEOR   | 11  7/6  4 |

|        | 34         |          Bits 2, 1 and 0: Destination bit

|*BIEOR  | 11  7/6  4 |

|        | 35         |

|*LDBT   | 11  7/6  4 |      Source/Destination Bit in binary form:

|        | 36         |

|*STBT   | 11  8/7  4 |      0 - 000   2 - 010   5 - 100    6 - 110

|        | 37         |      1 - 001   3 - 011   5 - 101    7 - 111

---------------------

Both  the  source  and  destination  bit  portions of the post-byte are

looked at by the 6309 as the actual bit NUMBER to  transfer/store.  Use  the

binary  equivilant  of the numbers (0 thru 7) and position them into the bit

area of the post byte.

Mnemonics Reference Table                                            Page 25

Logical Memory Operations

_________________________________________________________________________

|  Mnem  |   Immed.   |   Direct   |   Indexed  |  Extended  |  Inherent  |

|        |            |            |            |            |            |

|        | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # | OP  ~/~  # |

|--------+------------+------------+------------+------------+------------|

|*AIM    |            | 02   6   3 | 62   7+  3+| 72   7   4 |            |

|*EIM    |            | 05   6   3 | 65   7+  3+| 75   7   4 |            |

|*OIM    |            | 01   6   3 | 61   7+  3+| 71   7   4 |            |

|*TIM    |            | 0B   6   3 | 6B   7+  3+| 7B   5   4 |            |

-------------------------------------------------------------------------

Inter-Register Instructions               Transfer/Exchange and

__________________________________          Inter-Register Post Byte

|  Mnem  |   Forms    |  Register  |

|        |            |            |     _______________|_______________

|        |            | OP  ~/~  + |    |   |   |   |   |   |   |   |   |

|--------+------------+------------|    |     SOURCE    |  DESTINATION  |

|*ADCR   | R0,R1      | 10   4   3 |    |___|___|___|___|___|___|___|___|

|        |            | 31         |        HI NIBBLE   |  LOW NIBBLE

|*ADDR   | R0,R1      | 10   4   3 |

|        |            | 30         |

|*ANDR   | R0,R1      | 10   4   3 |             Register Field

|        |            | 34         |          (source or destination)

|*CMPR   | R0,R1      | 10   4   3 |

|        |            | 37         |      0000 - D (A:B)    1000 - A

|*EORR   | R0,R1      | 10   4   3 |      0001 - X          1001 - B

|        |            | 36         |      0010 - Y          1010 - CCR

| EXG    | R0,R1      | 1E  8/5  2 |      0011 - U          1011 - DPR

|*ORR    | R0,R1      | 10   4   3 |      0100 - S          1100 - 0

|        |            | 35         |      0101 - PC         1101 - 0

|*SBCR   | R0,R1      | 10   4   3 |      0110 - W          1110 - E

|        |            | 33         |      0111 - V          1111 - F

|*SUBR   | R0,R1      | 10   4   3 |

|        |            | 32         |

| TFR    | R0,R1      | 1F  6/4  2 |  The  results  of all  Inter-Register

|*TFM    | R0+,R1+    | 11  6+3n 3 |  operations are  passsed into R1 with

|        |            | 38         |  the exception of EXG which exchanges

|*TFM    | R0-,R1-    | 11  6+3n 3 |  the values of  registers and the TFR

|        |            | 39         |  block transfers.

|*TFM    | R0+,R1     | 11  6+3n 3 |

|        |            | 3A         |  The register field codes %1100 and

|*TFM    | R0,R1+     | 11  6+3n 3 |  %1101 are both zero registers.  They

|        |            | 3B         |  can be used as source or destination.

----------------------------------

Mnemonics Reference Table                                            Page 26

Indexed Address Modes and Post byte Information

__________________________________________________________________________

|                           Non-Indirect Modes                             |

|--------------------------------------------------------------------------|

|         Type            |     Forms     | Assembler | PostByte | +/+ | + |

|                         |               |   form    | OP code  | ~/~ | # |

|-------------------------+---------------+-----------+----------+-----+---|

| Constant offset from R  | No offset     |    ,R     | 1rr00100 |  0  | 0 |

|                         | 5 bit offset  |   n,R     | 0rrnnnnn |  1  | 0 |

|                         | 8 bit offset  |   n,R     | 1rr01000 |  1  | 1 |

|                         | 16 bit offset |   n,R     | 1rr01001 | 4/3 | 2 |

|-------------------------+---------------+-----------+----------+-----+---|

| Accumulator offset      | A - Register  |   A,R     | 1rr00110 |  1  | 0 |

| from R (Twos complement | B - Register  |   B,R     | 1rr00101 |  1  | 0 |

|*offset)                 | E - Register  |   E,R     | 1rr00111 |  1  | 0 |

|*                        | F - Register  |   F,R     | 1rr01010 |  1  | 0 |

|                         | D - Register  |   D,R     | 1rr01011 | 4/2 | 0 |

|*                        | W - Register  |   W,R     | 1rr01110 | 4/1 | 0 |

|-------------------------+---------------+-----------+----------+-----+---|

| Auto increment and      | Increment 1   |    ,R+    | 1rr00000 | 2/1 | 0 |

| decrement of R          | Increment 2   |    ,R++   | 1rr00001 | 3/2 | 0 |

|                         | Decrement 1   |    ,-R    | 1rr00010 | 2/1 | 0 |

|                         | Decrement 2   |    ,--R   | 1rr00011 | 3/2 | 0 |

|-------------------------+---------------+-----------+----------+-----+---|

| Constant offset from PC | 8 bit offset  |   n,PC    | 1xx01100 |  1  | 1 |

| (Twos complement offset)| 16 bit offset |   n,PC    | 1xx01101 | 5/3 | 2 |

|-------------------------+---------------+-----------+----------+-----+---|

|*Relative to W           | No Offset     |   ,W      | 10001111 |  0  | 0 |

|*(Twos complement offset)| 16 bit offset |   n,W     | 10101111 | 5/2 | 2 |

|* AutoIncrement W        | Increment 2   |   ,W++    | 11001111 | 3/1 | 0 |

|* AutoDecrement W        | Decrement 2   |   ,--W    | 11101111 | 3/1 | 0 |

|--------------------------------------------------------------------------|

|                             Indirect Modes                               |

|-------------------------+---------------+-----------+------------+---+---|

| Constant offset from R  | No offset     |  [ ,R]    |  1rr10100  | 3 | 0 |

|                         | 5 bit offset  |  [n,R]    |  Defaults to 8 bit |

|                         | 8 bit offset  |  [n,R]    |  1rr11000  | 4 | 1 |

|                         | 16 bit offset |  [n,R]    |  1rr11001  | 7 | 2 |

|-------------------------+---------------+-----------+------------+---+---|

| Accumulator offset      | A - Register  |  [A,R]    |  1rr10110  | 4 | 0 |

| from R (Twos complement | B - Register  |  [B,R]    |  1rr10101  | 4 | 0 |

|*offset)                 | E - Register  |  [E,R]    |  1rr10111  | 1 | 0 |

|*                        | F - Register  |  [F,R]    |  1rr11010  | 1 | 0 |

|                         | D - Register  |  [D,R]    |  1rr11011  | 4 | 0 |

|*                        | W - Register  |  [W,R]    |  1rr11110  | 4 | 0 |

|-------------------------+---------------+-----------+------------+---+---|

| Auto Increment and      | Increment 2   |  [,R++]   |  1rr10001  | 6 | 0 |

| decrement of R          | Decrement 2   |  [,--R]   |  1rr10011  | 6 | 0 |

|-------------------------+---------------+-----------+------------+---+---|

| Constant offset from PC | 8 bit offset  |  [n,PC]   |  1xx11100  | 4 | 1 |

| (Twos complement offset)| 16 bit offset |  [n,PC]   |  1xx11101  | 8 | 2 |

|-------------------------+---------------+-----------+------------+---+---|

| Extended indirect       | 16 bit address|   [n]     |  10011111  | 5 | 2 |

|-------------------------+---------------+-----------+------------+---+---|

|*Relative to W           | No Offset     |   [,W]    |  10010000  | 0 | 0 |

|*(Twos complement offset)| 16 bit offset |   [n,W]   |  10110000  | 5 | 2 |

|* AutoIncrement W        | Increment 2   |   [,W++]  |  11010000  | 3 | 0 |

|* AutoDecrement W        | Decrement 2   |   [,--W]  |  11110000  | 3 | 0 |

--------------------------------------------------------------------------

rr = X, Y, U or S      X = 00  Y = 01

xx = Doesn't care      U = 10  S = 11

+ and +   indicates the additional number of cycles and bytes for the

~     #   particular variation

Mnemonics Reference Table                                            Page 27

Register Descriptions

_________________________________________________________________________

| X  - 16 bit index register                                              |

| Y  - 16 bit index register                                              |

| U  - 16 bit user-stack pointer                                          |

| S  - 16 bit system-stack pointer                                        |

| PC - 16 bit program counter register                                    |

|*V  - 16 bit variable register (inter-register instructions only)        |

|*0  - 8/16 bit zero register   (inter-register instructions only)        |

|-------------------------------------------------------------------------|

| A  - 8 bit accumulator             |                                    |

| B  - 8 bit accumulator             |    Accumulator structure map:      |

|*E  - 8 bit accumulator             |      ----- ----- ----- -----       |

|*F  - 8 bit accumulator             |     |  A  |  B  |  E  |  F  |      |

| D  - 16 bit concatenated reg.(A B) |      -----------+-----------       |

|*W  - 16 bit concatenated reg.(E F) |     |     D     |     W     |      |

|*Q  - 32 bit concatenated reg.(D W) |      -----------------------       |

|------------------------------------|     |           Q           |      |

|*MD - 8 bit mode/error register     |      -----------------------       |

| CC - 8 bit condition code register | bit  31   24    15    8     0      |

| DP - 8 bit direct page register    |                                    |

-------------------------------------------------------------------------

* Indicates new registers in 6309 CPU.

Push/Pull Order of Stack

Pull order                  Push/Pull Post byte

|                 -------------------------------

\|/               | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

'                 -------------------------------

CC                 |   |   |   |   |   |   |   |____CCr

A\                 |   |   |   |   |   |   |________A

B/ D\ Q            |   |   |   |   |   |____________B

E\ W/              |   |   |   |   |________________DPr

F/                 |   |   |   |____________________X

DP                 |   |   |________________________Y

X-hi                |   |____________________________S/U

X-low               |________________________________PC

Y-hi

Y-low

U/S-hi

U/S-low

PC-hi

PC-low

.

/|\

|

Push order

Condition Code Register

-------------------------------

| E | F | H | I | N | Z | V | C |

-------------------------------

Entire flag____|   |   |   |   |   |   |   |____Carry flag

FIRQ mask________|   |   |   |   |   |________Overflow

Half carry____________|   |   |   |____________Zero

IRQ mask________________|   |________________Negative

The PSH(s,u) and PUL(s,u) instructions require one additional

cycle for each byte pushed or pulled.

Alan DeKok's addition to the above...

The new features of the 6309 are closely related to the changes in

design from the 6809.  The 6309 is micro-coded, which allowed the

designers to easily add new instrctions and registers.  It also has a

one byte pre-fetch 'cache', and an internal pipeline.  The 'cache'

enables the 6309 to execute

instructions like 'lsld' (2-bytes) in one clock cycle.  The design of

the 6809 series allows them to read one byte per clock cycle MAXIMUM, but there

is a catch.  Most instructions take more clock cycles to execute than

bytes they contain.  While the 6309 is performing internal

calculations, the 'cache' hardware goes and reads the next instruction

byte, leaving only one additional byte to be read to execute the 'lsld'.

Reading this byte requires one clock cycle, and then the instruction

is executed while the CPU fetches the next instruction.

The 6309 has a true 16-bit internal design.

e.g. the EXG instruction operates as

6809: read op-code

read inter-register byte (r0,r1)

r0_high -> temp_high

r0_low  -> temp_low

r1_high -> r0_high

r1_low  -> r0_low

r0_high -> r1_high

r0_low  -> r1_low

8 actions, 8 clock cycles.

6809: read op-code

read inter-register byte (r0,r1)

r0 -> temp

r1 -> r0

r0 -> r1

5 actions, 5 clock cycles.

The 6309 native mode instruction execution clock lengths can be mostly accounted

for by accounting for the pre-fetch cache and the internal 16-bit ALU.

TFM has some caveats. TFM r1-,r2- should NOT be used to setup the

stack, as it's a POST-decrement instruction, not PRE-decrement.

Watch out for TFM r1,r2+ if you're reading from a peripherial.

Why?  The TFM uses the 1-byte 'cache' as an internal buffer for the

byte that it's currently moving.  The TFM instruction is interruptible

(the only instruction that is), and code execution during the

interrupt will destroy the byte in the cache.

On returning from the interrupt, the TFM will read the FROM

address again to get the lost byte, which may be the wrong one.  The

visible effect of this is that block moves sometimes have a byte

missing from the middle, and everything after that byte shifted down

one address.

The W,E, and F registers do not have the full immediate addressing

mode capabilities that D,A, and B do.  SBC, AND, BIT, EOR, ADC, OR

with E,F,W are available only in register-register mode.  LSR, ROR,

ROL are available for W but not for E,F.  ASR, ASL, LSL, NEG do not

exist at all for W,E,F.

ASL can sort of be implemented by doing a ADDR R1,R1.  (see later)

You can also do things like 'leax u,x' by doing a ADDR u,x.

Sadly, many of the new 6309 instructions are useless in everyday

life.  The bit manipulation instructions are interesting, but slow and

mostly of limited value.  Same with much of the DIV/MUL higher math.  The

AIM, etc. are very useful, though.

Programmer recommendations

Try to stay away from using the W register.  It's got another pre-byte

(like instructions using 'Y' or 'S'), and is correspondingly larger

and slower.  E and F are best used mainly instead of pushing loop counters onto

the stack when you're running out of registers.

The V register is mostly pointless.  If you're doing context

switches, it isn't saved across interrupts unless you do so manually.

Shuffling data back and forth between other registers and V is a lot

of trouble.  Any math, etc. involving V is generally done much faster

using a real register.  After going through 1meg+ of 6309 assembly code

which is everything from an OS kernel to serial drivers to graphics

drivers, I've never seen a use for the V register.

Of course, you could put '$FFFF' into V, and have registers for

reg-reg addressing modes with bits all zero (0), and another with bits all

1 (V).

Pseudo-nops: tfr 0,0; exg 0,0

Extremely small software timing loops with large delays may be generated

by performing a 'LDW',and then 'TFM 0+,0+'.

Many programs can be executed in 6309 native mode by patching only

the IRQ code, if it accesses the stack.  A 'LDMD #$01' may be

performed as soon as your program starts executing, and will see an

immediate 10-15% speed increase.  Software timing loops must be

checked!

Opcodes Hitachi left out of the 6309: and some round-about equivalents

E/F/W

-----

ADCr:  ADCR 0,r

ANDr:  ; ANDR V,r

ASLr/LSLr:  ADDR r,r

ASRr

BITr

EORr

NEGr:  COMr INCr

ORr

SBCr:  SBCR Z,r

E/F

---

LSRr

ROLr: ADCR r,r

RORr

Q (Long word =W1:W0)

--------------------

ADDQ: ADDW W0; ADCD W1

SUBQ: SUBW W0; SBCD W1

ASLQ: ASLW   ; ROLD

ROLQ: ROLW   ; ROLD

LSRQ: LSRD   ; RORW

RORQ: RORD   ; RORW

ASRQ: ASRD   ; RORW

COMQ: COMD   ; COMW

NEGQ: COMD   ; COMW ; SBCR 0,D