Infrared Remote Control Receiver for CoCo I/II/III        ==================================================    All materials in this archive are released to the publicdomain. Use them at your own risk.        Purpose:        --------    To allow your CoCo to receive and interpret signals from aMitsubishi HS-U32 VCR infrared remote control. Use of other modelsshould be possible with changes to the software.        Contents of archive:        --------------------REMOTE.DOC -- this fileREMOTE.TXT -- 6809 MAC Assembler source fileREMOTE.LST -- 6809 MAC Assembler listing fileREMOTE.BAS -- BASIC driver program -- POKEs the assembly program                into memory and runs it        Hardware:        ---------    This program uses a Radio Shack #276-137 IR detector module(cost: all of $3.49) connected to the right joystick port.Connection requires only three wires: pin 3 of the module(nearest the metal case) to ground (pin 3 on the joystickconnector); pin 2 of the module to +5V for power (pin 5 on thejoystick connector); and pin 1 of the module to the joystick"fire button" input (pin 4 on the joystick connector). The metalcase of the module should be connected to ground also.    You can use an RS #274-003 5-pin DIN plug with any CoCo.If you have a CoCo III, you could also use an RS 274-020 6-pinDIN plug. Looking at the back of the plug, at the ends of thepins that you solder to, rotate it so the notch in the bodypoints straight up. Pin 5 (+5V) is the first pin to the left, orcounterclockwise, of the notch. Pin 4 (the "fire" button input)is the next, continuing left or counterclockwise. Pin 3 (ground)is the next; it is opposite the notch. The remaining pins arenot used in this project.    Plug the joystick connector into the right joystick port andturn on your CoCo. Pin 3 of the module and its case should be atground (0 volts). Pins 1 and 2 should be at about 5 volts.        Software:        ---------    The REMOTE.BAS program simply calls the machine-languageroutine, interprets the result, and displays the interpretation.Making it do something useful is up to you!    Note that the interpretation is based on my model of IRremote. Other Mitsubishi models may assign different functionsto some of these codes. Models from other manufacturers willprobably use a completely different encoding scheme (see below,"Further Experimentation").    The machine-language routine checks to see that the correctnumber of bits was received. If not, the USR value is set tozero. No other error checking is done.    The program runs at normal CoCo speed (0.895 MHz). It alsodisables interrupts, so the TIMER function and otherinterrupt-based functions are disabled.    Finally, note that the machine language routine will loopforever if it doesn't get a signal from the joystick button.Also, the BASIC program loops forever. Getting out of theprogram requires pressing the reset button, or pressing BREAKwhile the BASIC program is printing.        Theory of Operation:        --------------------    IR remotes send out bursts of IR light, each burstconsisting of a number of on/off pulses. The desired operation("PLAY", "RECORD", etc.) is encoded by the length and/or spacingof these bursts.    (Note: The rate at which on/off pulses occur within a burstis the pulse repetition frequency (PRF). Mitsubishi remotes usea PRF of 33.333 KHz (33,333 per second). This is relatively low;Sony remotes are three times as fast, 100 kHz. The Radio ShackIR detector module is supposed to receive PRFs of 40 kHz, butseems to have no trouble with much higher and lower PRFs. Thisis probably one reason Radio Shack chose to sell this particularmodel.)    The Radio Shack IR detector module recognizes these IRbursts. It's pin 3 is normally HIGH (5 volts). During an IRburst, it drops LOW (0 volts). Pin 3 is connected to the CoCojoystick "fire button" input, which is normally HIGH when thebutton isn't pressed and LOW when it is. So the presence orabsence of an IR burst is read as joystick button pressed or notpressed.    The machine-language routine waits for the joystick buttonbit to drop low, indicating the start of IR transmission. Itthen goes into a loop, reading the joystick button and storingit continuously. In order to allow sampling to occur as fast aspossible, the data is just stored; no analysis is done yet. Theloop exits when the memory (8 kilobytes) allocated for storageis filled. (Note that, since, in the CoCo's design, the joystickbutton is connected to a keyboard row, use of the keyboard mayinterfere with reception of IR signals and vice versa.)    Then, the 8 kilobytes of stored button states is analyzeddown to a few dozen bytes in the run-length table. The firstbyte in the run-length table is the number of entries. Eachfollowing byte is a run-length count. The first following byteis how long the initial "on" burst lasted. The second followingbyte is how long the first "off" gap lasted. The third followingbyte is how long the second "on" burst lasted. Lengths of "off"gaps and "on" bursts alternate to the end of the table.        Address         Contents        $7100           N = Size of table        $7101           LON1 = length of first "on" burst        $7102           LOFF1 = length of first "off" gap        $7103           LON2 = length of second "on" burst        $7104           LOFF2 = length of second "off" gap                ...        $7100+N         LONN = length of final STOP burst.    The routine then checks the size of the table. If N is not33, there was an error, and the return word is set to zero. If Nis 33, the return word has its bits set according to the lengthsof the "off" gaps. A gap more than 60 corresponds to a "1" bit;a gap less than 60 to a "0" bit. (Typically, short gaps areabout 30 and long gaps around 90 to 100; 60 was chosen as beinghalfway between.) The word is then returned to BASIC via the USRvalue.    The Mitsubishi word appears to consist of two bytes. Thefirst eight-bit byte is a device code. VCRs have a device codeof &HEA (decimal 234). Other devices -- TVs, CD players, etc. --have different codes. (TVs, for example, are &HE2.) The last bitof the second byte is a unit code. Newer Mitsubishi VCRs have a"VCR A/VCR B" switch. VCR A will only respond to the remote ifthe last bit is zero; VCR B only if it is one. The top 7 bits ofthe second word are the actual function codes ("PLAY", "STOP",etc.). These will differ somewhat between models, although thebasic functions will probably be the same. So there are at most128 different function codes.    Thus you have at most 128 (7 bits worth) different possiblecodes for a VCR remote, plus one bit of selection betweendifferent VCRs.         Further Experimentation:        ------------------------    Unfortunately, there are no current standards for theencoding of IR remote control signals. Every manufacturer doesit differently. So the REMOTE.BAS program, written for aMitsubishi remote, will probably be useless with other brandswithout modification.    Fortunately, practical and economic limits have forcedmanufacturers to do some things the same way. As far as I know,all IR remotes are digital: the IR signal is either on or off,with no other states. Therefore, manufacturers have two basicmethods of encoding: burst position modulation (BPM), and burstwidth modulation (BWM).    In BPM, each "on" burst is the same length as all theothers. Information is encoded by varying the length of the"off" period from one burst to the next. Mitsubishi uses asimple BPM method: "on" bursts are all about half a millisecondlong. A zero bit is indicated by a delay of the same lengthbefore the next "on" burst. A one bit is indicated by a delayabout 3 times as long. This means that a word with a lot of onebits will take longer to transmit than a word with mostly zerobits. Also, a transmitted word of 16 bits will consist of 17"on" bursts with the 16 "off" gaps between them, since it is the"off" gaps that carry the 16 bits of information. There will bea total of 33 entries in the run length table.    In BWM, the length of the burst itself changes to carry theinformation. The simplest case is just the mirror-image of theMitsubishi method, with all "off" gaps the same length, shortbursts for "zero," and long bursts for "one."    However, more complicated methods are possible and used. For example, some manufacturers use an extremely long burst andlong gap at the beginning of a word, with all the followingbursts and gaps being shorter. I suppose this allows thereceiver to wake up and get ready to receive data before theinformation-bearing bursts start to come in. Often in thesesystems, there is a special code for "repeat the previouscommand," consisting of the long start burst, a gap half aslong, and a single short burst. If you hold the "CHANNEL UP" keyon one of these remotes, the "CHANNEL UP" code is only sentonce, followed by a string of "repeat" codes.    If you want to use a non-Mitsubishi remote, you'll have tobreak the code yourself. There are too many possibilities for meto cover. Change line 220 in the BASIC driver program to "GOSUB41000" to see the actual run lengths. If the remote has numberkeys, try numbers that differ by one (e.g., "5" and "6"). Theirencoding may differ by just one bit, which may simplify figuringthe coding scheme (but don't count on it). Keep in mind that theword length may differ with different manufacturers. Good luck!##############################################################################################################